📘 AXI-Lite Protocol Overview

This document explains the AXI-Lite protocol as implemented in the axi_lite_fifo_wrapper module. AXI-Lite is a simplified version of the AXI protocol, designed for low-throughput control interfaces. It uses separate channels for read and write operations, each with a valid–ready handshake.

🔄 AXI-Lite Channel Summary

| Channel | Direction | Signals | Purpose |
| --- | --- | --- | --- |
| Write Address | Master → Slave | AWADDR, AWVALID, AWREADY | Send write address |
| Write Data | Master → Slave | WDATA, WVALID, WREADY | Send write data |
| Write Response | Slave → Master | BRESP, BVALID, BREADY | Acknowledge write completion |
| Read Address | Master → Slave | ARADDR, ARVALID, ARREADY | Send read address |
| Read Data | Slave → Master | RDATA, RRESP, RVALID, RREADY | Return read data and status |

Each channel uses a valid–ready handshake: a transaction occurs when both VALID and READY are high in the same cycle.

🧠 Handshake Timing

✅ Write Transaction

text

Cycle 1: AWVALID = 1, WVALID = 1

Cycle 2: AWREADY = 1, WREADY = 1 → handshake

Cycle 3: BVALID = 1

Cycle 4: BREADY = 1 → response accepted

✅ Read Transaction

text

Cycle 1: ARVALID = 1

Cycle 2: ARREADY = 1 → handshake

Cycle 3: RVALID = 1

Cycle 4: RREADY = 1 → data accepted

📦 FIFO Wrapper Address Map

| Address | Function | Description |
| --- | --- | --- |
| 0x00 | FIFO Push | Write data into FIFO |
| 0x04 | FIFO Pop | Read data from FIFO |
| 0x08 | FIFO Status | Read {count, full, empty} flags |
| Other | Invalid Access | Returns BAD_ADDR, triggers DECERR |

🧪 Response Codes

| Code | Meaning | Trigger Condition |
| --- | --- | --- |
| 2'b00 | OKAY | Valid transaction |
| 2'b10 | SLVERR | FIFO full (write) or empty (read) |
| 2'b11 | DECERR | Invalid address access |

🔍 Example Transactions

📝 FIFO Push

cpp

driver.write(0x00, 0xA5);  // Push value into FIFO

📤 FIFO Pop

cpp

uint32_t val = driver.read(0x04);  // Pop value from FIFO

📊 Status Read

cpp

uint32_t status = driver.read(0x08);  // Read full/empty/count

❌ Error Injection

cpp

driver.inject_error(0xDEADBEEF, 0xCAFEBABE, true, true);  // Trigger DECERR

📈 Waveform Snapshots

See docs/waveform_snapshots.md for annotated timing diagrams showing:

- Write and read handshakes

- FIFO full/empty behavior

- Error responses

📌 Related Files

- axi_lite_fifo_wrapper.sv – RTL implementation

- axi_lite_driver.cpp – C++ driver for transactions

- axi_lite_monitor.cpp – C++ monitor for responses and coverage

- coverage_report.md – Functional coverage bins and validation