📊 AXI-Lite FIFO Coverage Report

This document summarizes the functional coverage strategy for the axi_lite_fifo_wrapper module, as observed by the AxiLiteMonitor and tracked via CoverageTracker.

✅ Coverage Goals

The following coverage bins are tracked to ensure robust verification of the AXI-Lite FIFO interface:

1. Transaction Type Coverage

| Bin | Description | Trigger Condition |
| --- | --- | --- |
| read_count | Number of AXI-Lite read ops | ARVALID && ARREADY && RVALID && RREADY |
| write_count | Number of AXI-Lite write ops | AWVALID && AWREADY && WVALID && WREADY |

✅ Validated by: run_fifo_sequence() and burst sequences in the testbench.

2. Address Range Coverage

| Bin | Address Range | Description |
| --- | --- | --- |
| addr_low | 0x00–0x0F | FIFO push address |
| addr_mid | 0x10–0x7F | Reserved/unused (for stress) |
| addr_high | 0x80 and above | Invalid address range |

✅ Validated by: direct writes to 0x00, 0x10, 0x80, and 0xDEADBEEF.

3. Data Pattern Coverage

| Bin | Pattern | Description |
| --- | --- | --- |
| data_zero | 0x00000000 | All-zero write |
| data_ones | 0xFFFFFFFF | All-one write |
| data_alt | 0xAAAAAAAA mask | Alternating bits (e.g., 0xAAAAAAAA) |

✅ Validated by writing specific patterns in burst and error tests.

4. FIFO Depth Coverage

| Bin | Condition | Description |
| --- | --- | --- |
| fifo_empty | fifo_level == 0 | FIFO is empty |
| fifo_half | fifo_level == DEPTH / 2 | FIFO is half full (e.g., 8/16) |
| fifo_full | fifo_level == DEPTH | FIFO is full |

✅ Validated by pushing/popping known number of elements and sampling fifo_level.

5. Burst Length Coverage

| Bin | Length | Description |
| --- | --- | --- |
| burst_1 | 1 | Single transaction |
| burst_4 | 4 | 4-beat burst |
| burst_8 | 8 | 8-beat burst |

✅ Validated by calling burst_write() and burst_read() with varying lengths.

6. Error Injection Coverage

| Bin | Condition | Description |
| --- | --- | --- |
| err_invalid_addr | Address = 0xDEADBEEF or invalid | Simulates decode error |
| err_corrupt_data | Data XORed with 0xFFFFFFFF | Simulates corrupted write/read |

✅ Validated by inject_error() calls in the testbench.

🧪 Validation Strategy

- All bins are sampled via AxiLiteMonitor::sample() during simulation.

- Coverage is printed at the end of simulation using coverage.report().

- Waveform snapshots and logs are available in /waveforms/ and /logs/.